This is used for metric Bikes or American when the other adapter does not fit!

Cut the smaller white connector end above off and connect the red to your positive power source and black to your ground

Disregard the ( diagram only ) of the wiring on the manufacturers wiring directions, Its wrong!

I would advise to add a  7 Amp fuse and switch to your red wire for as an extra margin of safety!

The fuse and switch is not included with the system!

Speaker wires are pushed in 1st speaker top/ 2nd next down

Black speaker neg wire to right / Blue speaker Pos wire to left

Solid wires are positive and solid with stripes are Neg

Cut the plug off the end with two wires RED/ BLK and replace with terminals and a fuse

Red to positive / Black to negative

Inline adapter for most of American made bikes

Hopes this helps

NOTE: This system does not include a switch or fuse, I would advise to include the above with your installation or at least connect to a power source that shuts off when you shut the bike off!

This information was formatted by Cruising with Tunes and not the manufacturer to assist you with your installation.

Dave

www.cruisingwithtunes.com

PS: we now offer X-tra speakers for this system!

PS: To eliminate confusion per Lanzar Tech Support

( solid wires are positive ) and ( solid with stripes are neg )

These are the wires in the plastic connector from the amplifier

The speaker wires from the speakers are

( Blue is positive )

( Black is negative )